Model 680 ‘Super’

Type Certificate:	2A4

Date Certificated:	October 14th 1955

PCLM:	7 (2 at +95, 2 at +128, 3 at +168)

Number Built:	255 (but see Notes)

Certificated by:	Aero Design & Engineering Company

Serial numbers 242-1 through 647-254 	(254 aircraft)

Aero Design & Engineering Company (subsidiary of Rockwell-Standard Corp.)

Serial number 658-255	(1 aircraft)

Engines:	Lycoming GSO-480-A1A

Serial number 244-2 	(1 aircraft)

Lycoming GSO-480-A1A6

Serial numbers 242-1 & 288-3 through 591-229 	(228 aircraft)

Lycoming GSO-480-B1A6

Serial numbers 593-230 through 658-255 	(26 aircraft)

Propellers (hub/blade): 	Hartzell HC-83X20-2C/9333C

Serial numbers 242-1 through 482-152 & 484-154 through 658-255 	(254 aircraft)

Hartzell HC-83X20-2C1/9333C

Serial number 483-153 	(1 aircraft)

Fuel:	100/130 minimum grade aviation gasoline

Fuel consumption:	17 gallons per hour at 2,600r.p.m.

Fuel capacity:	225.5 U.S. gallons (854 litres) total	[Center tank 158.5gal. - usable 156gal.]

223.0 U.S. gallons (844 litres) usable	[Outboard tanks 33.5gal. each - all usable]

Oil capacity:	8.5 U.S. gallons (32.18 litres) total (4.25 gallons each engine)

8.5 U.S. gallons (32.18 litres) usable

| Weights & Balance: |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | lb | kg |  |  |
| Ramp Weight |  |  |  |  |
| MTOW | 7,000 | 3,175 |  |  |
| MLW |  |  |  |  |
| ZFW |  |  |  |  |
| Empty Weight (typical) | 4,420 | 2,005 | (Factory Empty Weight, with standard equipment) |  |
| Useful load |  |  |  |  |
| Basic Operating Weight |  |  |  |  |
| Payload |  |  |  |  |
| Maximum baggage | 350 | 159 |  |  |
| Maximum Wing Loading | 28.87 lb/sq.ft. | 140.94 kg/sq.ft. | 7,000lb / 242.5sq.ft. |  |
| Maximum Power Loading | 10.29 lb/hp | 4.67 kg/hp | 6.26 kg/kW | 7,000lb / 680hp |
| C.G. range | (+166.4) to (+175.8) - gear extended |  |  |  |
| Effect of retracting gear | +6655 in./lb. |  |  |  |
| Empty Weight C.G. range | None |  |  |  |
| Datum | Located 152 inches (12’ 8”) forward of wing leading edge at center section. |  |  |  |
| Levelling means | Longitudinal | Top of fuselage on centerline aft of wing trailing edge. |  |  |
|  | Lateral | Transverse beams at front or rear of baggage compartment floor. |  |  |

| Control Surface Movements: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | o up | o down | o right | o left | Remarks |
| Aileron | 23 +/-2 | 15 +/-2 |  |  |  |
| Flaps |  | 40 +2/-0 |  |  |  |
| Elevator | 30 +1/-0 | 10 +2/-0 |  |  | T.C.D.S. = 20 +/-1 up |
| Elevator tab | 2.5 +2/-0.5 | 20 +2/-0 |  |  | T.C.D.S. = 20 +2/-0 up |
| Rudder |  |  | 20 +2/-0 | 20 +2/-0 |  |
| Rudder tab |  |  | 26 +2/-0 | 26 +2/-0 |  |

| Dimensions: |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Feet/Inches | Inches | Meters | Remarks |
| Wing span | 44’ 0.70” | 528.70 | 13.429 |  |
| Wing chord at root | 8’ 4.00” | 100.00 | 2.540 |  |
| Wing chord at tip | 2’ 9.00” | 33.00 | 0.838 |  |
| Wing - ground to wingtip height |  |  |  |  |
| Wing Aspect Ratio |  |  |  | Airfoil section is NACA 23012 modified. |
| Taper Ratio |  |  |  | 3.03 |
| Wing dihedral |  |  |  | 4 degrees |
| Wing incidence at root |  |  |  | 3 degrees |
| Wing incidence at tip |  |  |  | minus 1 degree |
| Wing area | 242.50 sq.ft. |  | 22.529 m2 |  |
| Total Flap area | 21.20 sq.ft. |  | 1.970 m2 |  |
| Total Aileron area | 19.80 sq.ft. |  | 1.839 m2 |  |
| Overall Length | 35’ 5.00” | 425.00 | 10.795 |  |
| Fuselage Length | 35’ 0.56” | 420.56 | 10.682 |  |
| Overall Height | 14’ 6.00” | 174.00 | 4.420 |  |
| Ground to fuselage top height | 6’ 6.00” | 78.00 | 1.981 |  |
| Ground to fuselage bottom height |  |  |  |  |
| Total Fin area | 24.00 sq.ft. |  | 2.230 m2 | Airfoil section is NACA 0009. |
| Total Rudder area | 15.40 sq.ft. |  | 1.431 m2 |  |
| Tailplane span | 16’ 8.95” | 200.95 | 5.104 | Airfoil section is NACA 0011. |
| Ground to tailplane tip height |  |  |  |  |
| Tailplane dihedral |  |  |  | 10 degrees |
| Tailplane - angle of incidence |  |  |  | 0.75 degrees |
| Total Tailplane area | 33.06 sq.ft. |  | 3.071 m2 |  |
| Total Elevators area | 20.54 sq.ft. |  | 1.908 m2 |  |
| Wheel track (tread) | 12’ 11.0” | 155.00 | 3.937 |  |
| Wheelbase | 14’ 0.00” | 168.00 | 4.267 |  |
| Minimum ground turning radius | 76’ 3.50” |  | 23.254 |  |
| Cabin Length (Total) | 10’ 7.50” | 127.50 | 3.239 |  |
| Cabin Height | 4’ 5.00” | 53.00 | 1.346 |  |
| Cabin Width | 4’ 4.00” | 52.00 | 1.321 |  |
| Max. external fuselage width |  |  |  |  |
| Cabin door Height | 3’ 7.00” | 43.00 | 1.092 |  |
| Cabin door Width | 2’ 4.00” | 28.00 | 0.711 |  |
| Cabin door, Height to sill | 1’ 6.50” | 18.50 | 0.470 |  |
| Cabin volume | 177 cu.ft. |  | 5.012 m3 |  |
| Baggage compartment Length |  |  |  |  |
| Baggage compartment Height |  |  |  |  |
| Baggage compartment Width |  |  |  |  |
| Baggage door Height | 1’ 11.5” | 23.50 | 0.597 |  |
| Baggage door Width | 1’ 7.50” | 19.50 | 0.495 |  |
| Baggage door, Height to sill |  |  |  |  |
| Baggage compartment volume | 32 cu.ft. |  | 0.906 m3 |  |

| Performance: |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Feet | Meters |  | Remarks |
| Takeoff distance, over 50ft obstacle | 1,500 | 457 |  | or 1,575’ or 1,265’ (Jane’s) |
| Takeoff run |  |  |  |  |
| Short-field takeoff, over 50 ft obstacle |  |  |  |  |
| Short-field takeoff run |  |  |  |  |
| Landing distance, over 50ft obstacle | 1,605 | 489 |  | or 1,630’ or 1,450’ (Jane’s) |
| Landing run |  |  |  |  |
| Short-field landing distance, over 50ft obstacle |  |  |  |  |
| Short-field landing run |  |  |  |  |
| Maximum Operating Altitude |  |  |  | FAA Operational Ceiling |
| Service ceiling | 24,200 | 7,376 |  | or 25,567’ (Jane’s) |
| Single-engine ceiling | 15,000 | 4,572 |  |  |
|  | nm | sm | km |  |
| Range, maximum | 1,217 | 1,400 | 2,253 |  |
| Range, economy cruising - 55% S/L rated power | 1,286 | 1,480 | 2,382 | at 10,000’, 30 min. reserve |
| Range, maximum cruising power | 1,390 | 1,600 | 2,575 |  |
| Range, economy cruising power, max payload |  |  |  |  |
|  | mph | kts | km/hr |  |
| Manoeuvring | 160 | 139 | 257 | True Indicated |
| Max. structural cruising (Max. Operating Speed) (Vmo) | 210 | 182 | 338 | True Indicated |
| Never Exceed (Vne) | 270 | 235 | 435 | True Indicated |
| Flaps extended – Half (Vfe) | 150 | 130 | 241 | True Indicated |
| Flaps extended – Full | 130 | 113 | 209 | True Indicated |
| Landing gear extended | 180 | 156 | 290 | True Indicated |
| Maximum level speed (TAS) | 260 | 226 | 418 | at 10,000’ |
| Maximum cruising speed (TAS) |  |  |  |  |
| Cruising speed (TAS) - 70% power | 210 | 182 | 338 | at 10,000ft / 3,048m |
| Takeoff speed | 99 | 86 | 159 |  |
| Landing speed | 92.5 | 80 | 149 |  |
| Best rate-of-climb speed (Vy) | 120 | 104 | 193 |  |
| Best Angle of Climb speed (Vx) |  |  |  |  |
| Minimum Control speed (Vmc) |  |  |  |  |
| Stall Speed (Clean) (Vsi) | 79 | 69 | 127 |  |
| Stall Speed (Dirty) (Vso) | 71 | 62 | 114 |  |
|  | fpm | mpm |  |  |
| Initial Rate of Climb |  |  |  |  |
| Max. Rate of Climb at Sea Level | 1,625 | 495 |  |  |
| Max. Rate of Climb at Sea Level, single engine | 310 | 94 |  |  |

| Engines: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Characteristics | Piston, air cooled, Geared, Supercharged, with 6 horizontally Opposed cylinders. |  |  |  |
| Manufacturer | Lycoming |  |  |  |
| Engine Type | GSO-480 |  |  |  |
| Engine Variant | -A1A | -A1A6 | - B1A6 |  |
| Type Certificate Number |  | E-284 |  |  |
| Type Certificate Date |  | June 30th 1955 | May 9th 1957 |  |
| Applicable under STC No. |  | n/a | n/a |  |
| Maximum continuous HP |  | 320 | 320 |  |
| Maximum continuous RPM |  | 3,200 | 3,200 |  |
| Takeoff HP (5 minutes) * |  | 340 | 340 |  |
| Takeoff RPM |  | 3,400 | 3,400 |  |
| Weight (lb.) |  | 498 | 513 |  |
| Reduction Gear Ratio |  | 77 : 120 | 77 : 120 |  |
| Supercharging Ratio |  | 11.27 : 1 | 11.27 : 1 |  |
| Bore & Stroke (inches) |  | 5.125 x 3.875 | 5.125 x 3.875 |  |
| Displacement (cu.in.) |  | 479.7 | 479.7 |  |
| Compression Ratio |  | 7.3 : 1 | 7.3 : 1 |  |
| Carburetor Make & Model |  | Bendix (now P.A.C.) PS-7BD |  |  |
| Notes: | * no time constraint listed on T.C.D.S |  |  |  |
|  | The -B1A6 has crankcase oil jets for increased piston cooling, provisions for Supercharger inlet and an updraft carburetor. |  |  |  |
|  | The “6” in the designation indicates the crankshaft has five 3rd order and one 6th order torsional vibration dampers. |  |  |  |
|  | With the -B1A6 engines, the T.C.D.S. says “the oil cooler outlet gills must be relocated in accordance with Service Letter No. 62 and oil temperature gauge markings changed per Service Letter No. 63.” |  |  |  |

| Propellers: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Characteristics | Constant speed, hydraulic, with feathering and unfeathering features. |  |  |  |  |
| Manufacturer | Hartzell |  |  |  |  |
| Hub Model | HC-83X20-2C | HC-83X20-2C1 |  |  |  |
| Type Certificate Number | P-884 |  |  |  |  |
| Type Certificate Date | May 5th 1954 |  |  |  |  |
| Hub Material | Alloy Steel |  |  |  |  |
| Blade Model | 9333C |  |  |  |  |
| Blade Material | Aluminum Alloy |  |  |  |  |
| Number of Blades | 3 |  |  |  |  |
| Diameter (inches) | 93 |  |  |  |  |
| Diameter (meters) | 2.36 |  |  |  |  |
| Cut-off permitted | none |  |  |  |  |
| Prop tip/ground clearance (inches) | 16.69 |  |  |  |  |
| Prop tip/fuselage clearance (inches) | 5.50 |  |  |  |  |
| Distance between propeller centers | 153” |  |  |  |  |
| Pitch setting at 30" Station - Low | 17o |  |  |  |  |
| - High |  |  |  |  |  |
| - Feathered | 83o |  |  |  |  |
| Max. Hub & Blade weight (lb.) | 98 |  |  |  |  |
| HP - Maximum continuous | 320 |  |  |  |  |
| HP - Maximum Takeoff | 340 |  |  |  |  |
| RPM - Maximum continuous | 2,060 |  |  |  |  |
| RPM - Maximum Takeoff | 2,180 |  |  |  |  |

| Engine & Propeller combinations: |  |  |
| --- | --- | --- |
| Engines | Propellers (Hub / Blade) | Factory or other |
| Lycoming GSO-480-A1A | (not known) | Factory |
| Lycoming GSO-480-A1A6 | Hartzell HC-83X20-2C / 9333C | Factory |
|  | Hartzell HC-83X20-2C1 / 9333C | Factory |
| Lycoming GSO-480-B1A6 | Hartzell HC-83X20-2C / 9333C | Factory |
|  | Hartzell HC-83X20-2C1 / 9333C | Field upgrade |
| Note: In a handful of cases just ‘HC-83X20-2’ or HC-83X20-2A have been quoted in FAA paperwork. |  |  |

Notes:

255 Models 680 were physically built. Three were subsequently converted to other Models, as follows:

Serial 242-1 was converted to a Model 680E, and re-serialled with Model 680E unit number -102.

Serial 624-250 was also converted to a Model 680E, and re-serialled as unit number -9.

Serial 501-171 was converted to the first Model 720 Alti-Cruiser and re-serialled 720-501-1.

No initial Engine & Propeller data is held for serials 334-27, 471-141 and 501-171. The first two were U.S. military and the Warranty File data is sketchy. Serial 501-171 was, of course, re-worked as 720-501-1 and did not appear to have a CofA as a Model 680.

No initial Propeller data is held for serials 242-1 and 335-28.

--------------------------------------------------------------------oooOOOooo------------------------------------------------------------------